Action Points from Energy Audit

Current energy costs are £3,500 per annum for oil and £1,500 per annum for electricity.

Lighting

Question regarding Main Hall lights T8 or T12, checked and are T8 not T12, so good.

Replace all tungsten bulbs with modern fluorescent/LED equivalents, foyer & cupboards 9 in total.

Replace all tungsten halogen lights in toilets, main hall and bar area with LED type, 36 in total.

Replace 2 x T12 lamps in the kitchen and office to T8s, 4 in total.

Windows

Change to triple glazed fittings when the windows need replacement. In the meantime seal frames with silicone or tape if leaking.

Heating Controls

Separate zone on the heating system for upstairs & changing rooms.

Better controls on a timed basis for the building.

Insulation

Main hall room, void above suspended ceiling to be insulated,

Meeting room upstairs, void above suspended ceiling to be insulated,

Alternative Heating System

Install a new condensing boiler if grants are successful.

Two grants of £3,000 have been awarded; need to verify process and when payments are likely to be made. One quote in so far of £5,750, need two more quotes. Existing boiler is failing to start up, so it’s now getting critical.

Other Renewables

Investigate other sources of heating and power generation, such as biomass, heat pumps, wind turbines, solar heating, solar PV etc.

Biomass calculations and quotation received from EGS, costs are around £30,000 not including outbuilding costs. Nationwide do finance this type of build and use the heat tariff as their repayment mechanism. Needs more discussion and investigate other Halls that have already implemented this system.

Photo Voltaic calculations received from EGS, costs around £15,000, similar finance deals available but more attractive feed in tariffs may lend itself to local investors, needs more discussion.